WHEELER/WILD WILLY CONVERSION

Step 1- refer to step 11 and side bar of page 9 of the manual and rear of CVA pack

Use CVA dampers in place of the standard friction dampers. Refer to relevant sections on how to build. Combine with On Road Spring Set using red on the front and blue on the rear.

Step2- refer to step 9 of the manual and pic 1

Invert the front lower wishbone so as now it is now upside down. This will lengthen the wheelbase.  The flat side of the wishbone will now be facing forwards.

Step 3- refer to steps 9 and 10 of the manual, and pic 1

Remove the upright and replace it with part B9 (this part is originally unused).  Insert the rear MA10 axles into the front uprights using the appropriate bearings. Use the MB12 steering tie rods in place of MA4 to attach the steering knuckles to the C-hub.  On the rear side, use a 3mm nut. Then on the front, place a 5mm ball collar. Attach the dampers, then another 3mm nut, and then tighten. The upper arm, C3, is now installed on the rear side of the C-hub, and thus the MA1 step screw is screwed in from the rear. When later attaching the wheels, use E1 hexagonal drive hubs with pins.

Step 4- refer to steps 3and 4 of the manual

Install TG10 wheel axles at the rear. Use B14 hexagonal drive hubs (originally unused) and pins. It is advised to install aTL-01 stabiliser set before reattaching the wheelie bar/wheel and dampers.

Some trimming will be needed to complete Step 4.

Step 5- refer to steps 7 and 10 of the manual

Replace the servo horn E5, and replace with part B10 (originally unused). Parts C6 (originally unused) are used for steering tie rods. These are connected using 3x14 step screws.

Wheels used a RC Boys Voltec F+R rims with RC Boy F+R tires and inserts.

As the wheels are now smaller diameters, it is suggested using a 20T pinion in conjunction with a high revving motor, such as a Dyna Run Motor.  Therefore 1150 bearings through out chassis and gearbox are strongly recommended.

Attaching body mounts and body

The chassis interferes with the body in some places so will need to be trimmed in certain places.

These include; the wheelie bar, the two knobs on the battery holder to be removed, etc.

Referring to pic 2, 3 and 4, the body mounts consist of C8 and C20 on the rear and C5 (originally unused) on the front, using C2 parts of the now unused damper either side.